CSA07 – Computer Networks

Assessment Tool 2 – Scenario Based Questions

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology.

# Question 1 – Smart Campus Subnetting (192.168.100.0/24)

Given block: 192.168.100.0/24, subnetted using VLSM, allocated in descending order of size so each block starts on a valid boundary:

- School of Computing → /25 (128 addresses)

- School of Electronics → /26 (64 addresses)

- School of Mechanical Engineering → /27 (32 addresses)

## 1 & 2. Network / Broadcast Address and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| School of Computing | /25 (255.255.255.128) | 192.168.100.0 | 192.168.100.127 | 192.168.100.1 – 192.168.100.126 |
| School of Electronics | /26 (255.255.255.192) | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 – 192.168.100.190 |
| School of Mechanical Eng. | /27 (255.255.255.224) | 192.168.100.192 | 192.168.100.223 | 192.168.100.193 – 192.168.100.222 |

## 3. Usable Host Addresses

| Department | Total Addresses | Usable Hosts (2^n − 2) |
| --- | --- | --- |
| School of Computing (/25) | 128 | 126 |
| School of Electronics (/26) | 64 | 62 |
| School of Mechanical Eng. (/27) | 32 | 30 |

## 4. Remaining Unused IP Address Range

Total addresses used so far: 128 + 64 + 32 = 224, out of 256 in the /24 block.

Unused block: 192.168.100.224 – 192.168.100.255  (a /27-sized block of 32 addresses, available for future departments).

# Question 2 – Company Headquarters Subnetting (10.10.0.0/16)

Given block: 10.10.0.0/16, VLSM allocated largest-to-smallest:

- Software Development → /24 (256 addresses)

- Quality Assurance → /25 (128 addresses)

- Human Resources → /26 (64 addresses)

- Executive Management → /27 (32 addresses)

## 1 & 2. Network / Broadcast Address and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| Software Development | /24 | 10.10.0.0 | 10.10.0.255 | 10.10.0.1 – 10.10.0.254 |
| Quality Assurance | /25 | 10.10.1.0 | 10.10.1.127 | 10.10.1.1 – 10.10.1.126 |
| Human Resources | /26 | 10.10.1.128 | 10.10.1.191 | 10.10.1.129 – 10.10.1.190 |
| Executive Management | /27 | 10.10.1.192 | 10.10.1.223 | 10.10.1.193 – 10.10.1.222 |

## 3. Usable Hosts per Subnet

| Department | Total Addresses | Usable Hosts |
| --- | --- | --- |
| Software Development (/24) | 256 | 254 |
| Quality Assurance (/25) | 128 | 126 |
| Human Resources (/26) | 64 | 62 |
| Executive Management (/27) | 32 | 30 |

## 4. Unused IP Address Range Remaining

Addresses consumed: 256 + 128 + 64 + 32 = 480, from 10.10.0.0 through 10.10.1.223, inside the full /16 block of 65,536 addresses.

Unused range: 10.10.1.224 – 10.10.255.255  (the remainder of the /16 block, available for future expansion).

# Question 3 – IPv6 Migration for Smart Hospital

Given network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

## 1. Compressed Notation

2001:db8:abcd::

Leading zeros in each group are dropped (0db8 → db8), and the longest run of consecutive all-zero groups is replaced once with "::".

## 2. Expanded (Full) Notation

2001:0db8:abcd:0000:0000:0000:0000:0000

## 3. Network Prefix (/64)

The first 64 bits (first four hextets) form the network prefix, and the last 64 bits form the Interface ID:

Network Prefix: 2001:0db8:abcd:0000::/64

## 4 & 5. Total Number of Host (Interface) Addresses in the /64 Subnet

A /64 prefix leaves 128 − 64 = 64 bits for host (interface) addressing.

Total possible addresses = 2^64 = 18,446,744,073,709,551,616 addresses.

Unlike IPv4, IPv6 does not reserve a separate network/broadcast address from this count in the same way — effectively all 2^64 addresses in the subnet are available for host interfaces, which is why a single /64 is the standard allocation for one LAN segment regardless of actual device count.

# Question 4 – ISP Core Router Packet Forwarding

Routing table entries: 192.168.1.0/24, 192.168.2.0/24, 192.168.3.0/24

Destination IP of incoming packet: 192.168.2.45

## 1. Destination Subnet

192.168.2.45 falls within the range 192.168.2.0 – 192.168.2.255, so it belongs to subnet 192.168.2.0/24.

## 2. Matching Routing Table Entry

192.168.2.0/24 — this is the longest/only prefix match for the destination address.

## 3. Next-Hop Network / Interface

The router forwards the packet out of the interface configured for the 192.168.2.0/24 network (the interface directly connected to, or with a next-hop toward, that subnet).

## 4. Default Route (No Match Case)

If the destination does not match any entry, the router forwards the packet using the default route 0.0.0.0/0 (the gateway of last resort). If no default route is configured, the packet is dropped and an ICMP "Destination Unreachable" message is sent back.

# Question 5 – Administration & Finance LAN via Layer-3 Router

Administration LAN → 192.168.10.0/26   |   Finance LAN → 192.168.10.64/26

## 1 & 2. Network / Broadcast Address and Valid Host Range

| LAN | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- |
| Administration | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| Finance | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |

## 3. Suggested IP Addresses for Three Computers

| LAN | PC-1 | PC-2 | PC-3 |
| --- | --- | --- | --- |
| Administration | 192.168.10.2 | 192.168.10.3 | 192.168.10.4 |
| Finance | 192.168.10.66 | 192.168.10.67 | 192.168.10.68 |

(Note: .1 and .65 are reserved for the gateways; any address within each valid host range other than the gateway works.)

## 4. Default Gateway for Each LAN

| LAN | Default Gateway |
| --- | --- |
| Administration | 192.168.10.1 |
| Finance | 192.168.10.65 |

Hosts in the Administration LAN send inter-subnet/Internet-bound traffic to 192.168.10.1, and hosts in the Finance LAN use 192.168.10.65 as their gateway; the Layer-3 router routes between the two subnets.