CSA07 – Computer Networks

Assessment Tool 3 – Annexure A: Debugging & Optimisation Task

Model Answers – CO2 (Subnetting, IP Addressing & Routing)

# Question 1 – IPv4 Subnetting Fault (/26)

Given: IP Address 192.168.10.65, Subnet Mask 255.255.255.192 (/26), Default Gateway 192.168.10.1. The workstation is expected to sit in the subnet 192.168.10.0/26 but cannot communicate.

## 1. Identifying the subnet

A /26 mask gives a block size of 256 − 192 = 64. The /26 subnets of 192.168.10.0/24 are:

| Subnet | Network Address | Broadcast Address | Usable Host Range |
| --- | --- | --- | --- |
| Subnet 1 | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| Subnet 2 | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |
| Subnet 3 | 192.168.10.128 | 192.168.10.191 | 192.168.10.129 – 192.168.10.190 |
| Subnet 4 | 192.168.10.192 | 192.168.10.255 | 192.168.10.193 – 192.168.10.254 |

192.168.10.65 falls inside Subnet 2 (192.168.10.64/26), NOT inside 192.168.10.0/26 as the administrator assumed.

## 2. Network address, broadcast address, valid host range (for the host's actual subnet, 192.168.10.64/26)

Network Address: 192.168.10.64

Broadcast Address: 192.168.10.127

Valid Host Range: 192.168.10.65 – 192.168.10.126 (62 usable hosts)

## 3. Is the configured IP and gateway valid?

- IP 192.168.10.65 is a valid host address — but only within the 192.168.10.64/26 subnet, not within 192.168.10.0/26.

- Gateway 192.168.10.1 is a valid host address — but it belongs to the 192.168.10.0/26 subnet (range .1–.62).

- Result: the host (.64/26) and the gateway (.0/26) sit in two different subnets, so the workstation cannot reach the default gateway and therefore cannot reach any other network or even some hosts in the intended subnet.

## 4. Configuration errors identified

- Subnet mismatch: the host IP (192.168.10.65) does not belong to the subnet the administrator intended (192.168.10.0/26).

- Gateway mismatch: the configured gateway (192.168.10.1) is unreachable from the host because it lies in a different /26 subnet.

- No routing error exists on the LAN side — this is a pure addressing/subnet-boundary error.

## 5. Optimised (corrected) configuration

Two equally valid fixes — pick whichever matches the intended subnet:

Option A – Keep the host in 192.168.10.0/26 (match the gateway):

IP Address: 192.168.10.10  (any free address in 192.168.10.1 – 192.168.10.62)

Subnet Mask: 255.255.255.192 (/26)

Default Gateway: 192.168.10.1

Option B – Keep the host at .65 and move it into its real subnet (.64/26):

IP Address: 192.168.10.65

Subnet Mask: 255.255.255.192 (/26)

Default Gateway: 192.168.10.126  (last usable address in 192.168.10.64/26, reserved as the gateway)

Option A is the recommended fix since it keeps the stated target subnet (192.168.10.0/26) unchanged and only corrects the host's IP address.

## 6. Usable host addresses after correction

For a /26 subnet: 2^(32−26) − 2 = 2^6 − 2 = 64 − 2 = 62 usable host addresses (two addresses reserved for network and broadcast).

# Question 2 – IPv6 Addressing Fault (/64)

Given: Device A = 2001:db8::ff00:42:8329/64, Device B = 2001:db8:0:0:1::1/64, both on the same physical network, communication fails.

## 1. Fully expanded notation

The :: shorthand is expanded to restore all 8 groups (16 bits each):

| Device | Compressed | Fully Expanded |
| --- | --- | --- |
| Device A | 2001:db8::ff00:42:8329 | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Device B | 2001:db8:0:0:1::1 | 2001:0db8:0000:0000:0001:0000:0000:0001 |

## 2. Network prefix of each device (/64 = first 4 groups / 64 bits)

Device A prefix: 2001:0db8:0000:0000 :: /64

Device B prefix: 2001:0db8:0000:0000 :: /64

## 3. Do both devices belong to the same subnet?

Yes. Comparing the first 64 bits of both addresses, the prefixes are identical (2001:db8::/64). Addressing-wise, both devices DO belong to the same /64 subnet — there is no subnet/prefix mismatch, which is the opposite of what is normally assumed in this type of question.

## 4. Addressing / prefix mismatch identified

There is no prefix mismatch — the root cause of the communication failure must be sought outside of IPv6 addressing, for example:

- Duplicate Address Detection (DAD) conflict or an incomplete Neighbour Discovery Protocol (NDP) cache entry.

- A Layer‑2 fault: the two devices' switch ports placed in different VLANs despite sharing the same logical /64.

- A firewall / ACL blocking ICMPv6 (Neighbour Solicitation / Advertisement) messages needed for address resolution.

- Interface ID typo causing an unintended address clash with another host on the segment.

## 5. Optimised IPv6 configuration

To remove any ambiguity and make the allocation easy to manage, assign clear, sequential addresses from the same /64 prefix instead of ad‑hoc interface IDs:

Device A: 2001:db8::1/64

Device B: 2001:db8::2/64

Both remain inside the 2001:db8::/64 subnet, and the simplified host IDs remove any risk of transcription error. In parallel, Layer‑2/NDP connectivity (VLAN membership, switch port, firewall ICMPv6 rules) should be verified since the addressing itself was not the fault.

## 6. Usable host addresses in the corrected /64 subnet

A /64 subnet has 128 − 64 = 64 host bits, giving 2^64 (≈ 1.8 × 10^19, i.e. 18,446,744,073,709,551,616) possible interface addresses — for all practical purposes an unlimited address pool for a single LAN segment.

# Question 3 – VLSM Optimisation of 192.168.1.0/24

The /24 network has been split using /26, /27 and /28 masks, causing some departments to run out of addresses while others waste large blocks.

## 1 & 2. Network address, broadcast address, host range, and usable/unused counts per mask

| Mask | Block Size | Usable Hosts | Example Network/Broadcast | Host Range |
| --- | --- | --- | --- | --- |
| /26 | 64 | 62 | 192.168.1.0 / 192.168.1.63 | 192.168.1.1 – 192.168.1.62 |
| /27 | 32 | 30 | 192.168.1.64 / 192.168.1.95 | 192.168.1.65 – 192.168.1.94 |
| /28 | 16 | 14 | 192.168.1.96 / 192.168.1.111 | 192.168.1.97 – 192.168.1.110 |

Usable hosts formula: 2^(32 − prefix) − 2. Any department whose actual headcount is well below its block's usable-host figure is wasting addresses; any department whose headcount exceeds the usable-host figure is the one reporting shortages.

## 3. Inefficient allocations identified

- Fixed-size subnetting (classful-style, same mask for every department) forces every department onto the same block size, so a department needing 70 hosts cannot fit in a /27 (30 hosts) while a department needing only 5 hosts wastes 57 addresses in a /26 (62 hosts).

- Departments were evidently assigned masks without first ranking them by actual host requirement, leading to the reported shortage in some and surplus in others.

## 4. Debugging the addressing plan

- Error: subnet sizes were chosen independently of real headcount instead of being sized to need.

- Error: no contiguous, size-ordered allocation plan was used, risking overlapping or fragmented ranges as more subnets are added.

## 5. Recommended VLSM re-allocation

Rule: allocate the largest host requirement first, then progressively smaller blocks, so no address space is left stranded between subnets.

| Department (example sizing) | Hosts Needed | Subnet Assigned | Network/Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- |
| Dept A (largest) | up to 60 | 192.168.1.0/26 | 192.168.1.0 / .63 | 62 |
| Dept B | up to 25 | 192.168.1.64/27 | 192.168.1.64 / .95 | 30 |
| Dept C | up to 12 | 192.168.1.96/28 | 192.168.1.96 / .111 | 14 |
| Dept D | up to 12 | 192.168.1.112/28 | 192.168.1.112 / .127 | 14 |
| Remaining space | — | 192.168.1.128/25 | 192.168.1.128 / .255 | 126 (reserved for growth) |

This VLSM scheme (largest-first, contiguous allocation) matches each block to real demand, removes wasted addresses and leaves a clean, unused /25 block for future departments — the standard optimisation for the reported imbalance.

# Question 4 – IPv6 Addressing Fault (/64) — Repeat of Question 2

This question is identical to Question 2 (same Device A and Device B addresses, same /64 mask, same symptom). The analysis and corrected configuration are therefore the same as above:

- Fully expanded: Device A = 2001:0db8:0000:0000:0000:ff00:0042:8329, Device B = 2001:0db8:0000:0000:0001:0000:0000:0001.

- Network prefix of both devices: 2001:0db8:0000:0000::/64 — identical, so both devices are already in the same subnet.

- No addressing/prefix mismatch exists; the fault lies outside IPv6 addressing (DAD/NDP, VLAN, or firewall/ACL blocking ICMPv6).

- Optimised configuration: Device A = 2001:db8::1/64, Device B = 2001:db8::2/64.

- Usable host addresses in the /64 subnet: 2^64 ≈ 1.8 × 10^19 — effectively unlimited for one LAN segment.

# Question 5 – Routing Table / Longest-Prefix-Match Fault

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route (0.0.0.0/0) | 10.0.0.1 |

Symptom: packets for 192.168.2.100 are not reaching the intended network.

## 1. Matching entry using longest-prefix match

192.168.2.100 matches two entries: 192.168.2.0/24 (a specific /24 match) and the Default Route 0.0.0.0/0 (a 0‑bit match that matches everything). The router always prefers the entry with the longest (most specific) prefix, so the correct match is 192.168.2.0/24.

## 2. Next hop selected for forwarding

For a correctly functioning router, the packet should be forwarded to next hop 10.0.0.3 (the next hop listed against 192.168.2.0/24), not to the default route's 10.0.0.1.

## 3. Does the destination belong to the intended subnet?

Yes — 192.168.2.100 lies within 192.168.2.0/24 (valid host range 192.168.2.1 – 192.168.2.254), so the destination address itself is valid for that subnet.

## 4. Routing configuration errors identified

- If packets are not reaching 192.168.2.0/24 despite a correct, more-specific route being present, the likely faults are: the static route to 10.0.0.3 was removed/not installed in the routing table (forcing a fallback to the default route 10.0.0.1), the interface/next-hop 10.0.0.3 is down or unreachable (no reply from that next hop), or an ACL/firewall rule on the router is filtering traffic to 192.168.2.0/24.

- Any of these causes the router to silently fall back to the default route and send the traffic to 10.0.0.1 instead of the correct next hop 10.0.0.3, so it never reaches 192.168.2.0/24.

## 5. Optimising the routing table

- Verify and, if missing, re-add the static route: ip route 192.168.2.0 255.255.255.0 10.0.0.3.

- Confirm Layer‑3 reachability to 10.0.0.3 (ping/ARP) and that the link/interface facing it is up.

- Replace brittle static routes with a dynamic routing protocol (e.g., OSPF) so a failed next hop is automatically detected and an alternate path is used.

- Remove any unnecessary default-route dependency for internal subnets so misconfigured/missing specific routes fail loudly instead of silently falling back to the default route.

## 6. Route used when no destination entry matches

If no specific entry matches a destination, the router falls back to the Default Route (0.0.0.0/0), forwarding the packet to next hop 10.0.0.1.